Hemas WMS Slotting Plan and Seed Data (Raw + Packing Materials)

Generated: 2026-02-18 19:07

Purpose

This document explains how to convert your current on-hand quantities into handling units (drums, pallet loads, etc.) and estimate how many bin locations and rack bays are needed under a simple 5-level, 2-bin-per-level bay layout. It also provides database seed tables for a WMS: Product Catalog (materials only), Inventory Master, BOM mapping, and a per-item location requirement calculation.

Key idea (what your sheet is really saying)

In your Hemas sheet, 'Quantity' is in the material base UOM (often kg/L/pcs). 'Stacking quantity' is treated as the quantity per handling unit (e.g., kg per drum, or kg per pallet of bags, or pcs per pallet). The number of handling units is:
  HandlingUnits = ceil(Quantity / StackingQuantity).

Rack layout assumed

- 1 bay has 5 levels; each level has 2 bin locations (10 bins per bay).

- Each bin location is treated as one pallet position or one drum-rack position (depending on item type).

Level weight limits assumed (edit these to match your rack spec)

These are typical illustrative limits where upper levels carry less load.

| Level | Max weight per bin (kg) | Reason |
| --- | --- | --- |
| 1 | 1500 | Higher levels have reduced beam capacity |
| 2 | 1200 | Higher levels have reduced beam capacity |
| 3 | 900 | Higher levels have reduced beam capacity |
| 4 | 700 | Higher levels have reduced beam capacity |
| 5 | 500 | Higher levels have reduced beam capacity |

Handling unit assumptions used

These are not 'liquid capacity'. They are storage/handling assumptions for slotting. Where possible, we rely on standard packaging dimensions as reference and then apply conservative WMS rules.

References used for standard dimensions (for documentation):

- 200L drum dimensions (typical): https://en.wikipedia.org/wiki/Drum_(container)

- EPAL Euro pallet sizes: https://www.epal-pallets.org/eu-en/media-centre/product-sheets

- Example of 25kg bag palletization (45 bags per pallet): https://cancarb.com/wp-content/uploads/2020/05/packaging-25kg-paper-2.pdf

Storage rules applied (simple & realistic for WMS)

1) DRUM_RACK items: 1 drum per bin location (conservative).
2) PALLET_* items: 1 pallet per bin location (standard pallet position).
3) If the handling unit weight exceeds a level’s max weight, that level cannot be used.
4) Allocation is bottom-up: we place items on the lowest feasible level to respect weight capacity.

How bins needed are calculated

Step A: Convert on-hand quantity to handling units:
  HU_count = ceil(Quantity / HU_size).
Step B: Determine units-per-bin at each level based on weight:
  units_per_bin = floor(level_max_weight / HU_weight).
Step C (WMS rule): cap units_per_bin to 1 for pallet positions and 1 for drum racks.
Step D: bins_needed = ceil(HU_count / units_per_bin).
Step E: bays_needed = ceil(bins_needed / 10) because each bay has 10 bins.

What you can tune (recommended)

- Update level_max_weight to match your racking supplier spec.
- If your operation stores multiple drums on one pallet position, change DRUM_RACK capacity from 1 to 2 (or 4) and re-run.
- If some SKUs require temperature zones or hazmat segregation, add an 'allowed_zone' field and filter bins by zone.
- If your stacking quantity is not per handling unit, replace it with a dedicated HU conversion factor.

Outputs delivered

Excel workbook: hemas_wms_slotting_plan_and_seed_tables.xlsx
Sheets:
- Product_Catalog_Materials: raw + packing product master (no finished goods inventory)
- Inventory_Master: realistic ROP, safety stock, MOQ, lead time, and on-hand qty
- Rack_Capacity_Plan: level/bin capacities (edit here first)
- Location_Requirements: bins and bays needed per SKU + chosen level
- BOM_Mapping: FG-to-material mapping for production planning (FG not stored in inventory)