Instruction to use Stacker

Operators must be qualified

- Operating stacker should only be done by individuals who have been trained properly.

Appropriate clothing must be worn.

- It needs to be ensured that operators wear the appropriate safety work wear; usually consisting of a hard hat, safety shoes and hi-visibility jackets.

- The work wear must be reasonably fitted as any loose clothing can get caught on machinery.

- Don’t operate/hold any of the controls when your hands have grease on them; it may cause them to slide off and cause an accident.

Examine Equipment before use

- Operators should do a routine check of the equipment before driving them.  Some things you should check for any faults are brakes, steering, controls and warning devices.

- If there are any noted damages or problems management should be notified and the stacker should not be operated if it needs to be repaired.

- Always consider the ‘journey’s end’ of a load before picking it up.  A convenient position of a load from pick up may not be convenient for stacking.

Consider the surrounding environment

- Whilst operating a stacker you must pay attention and follow any work site rules and guidelines.

- The operator must only drive the equipment in the machinery’s designated roadways.

- Observe all signs, especially those on maximum permitted floor loadings and clearance heights.

- Be aware of the height of the load when entering or exiting buildings.

- Be careful when operating a stacker near the edge of a loading dock or ramp - the stacker can fall over the edge - keep a safe distance from the edge.

- Do not operate on bridge plates, unless they can support the weight of the stacker and load.

Operate at a safe speed

- Never proceed past the speed limit.

- Take corners and any turns slowly to minimize risk of tipping.

- Make any changes in direction or any stops gradually and slowly.

Avoid Hazards

- Steer clear of any bumps or uneven ground surfaces along with slippery conditions.

- Steer clear of loose ground objects which could cause loss of control over the equipment or a load to move around.

- Use the horn when closing in on a corner or doorway/entrance and around people to alert pedestrians or other stacker operators of your whereabouts to avoid any unnecessary collision.

- Keep a safe distance from other trucks in case they move in an unpredictable manner.

- Make sure that you always have enough space to stop safely.

Ensure your load is stable and secure

- Check the loads carefully before moving them for stability and damage.

- Check for any overhead objects before lifting or stacking loads.

- Do not lift or move loads that are not safe or stable.

- Make sure loads are correctly stacked and positioned across both forks.

- Stack the load on the pallet or skid safely and correctly.

- Use securing measures such as ropes or bindings if required.

Make sure you have clear visibility

- Operate the Stacker in reverse when it improves visibility; except when moving up ramps.

- It is important to make sure you can see the racking clearly in which you are positioning your load.

- If visibility is poor do not continue driving; in some circumstances you may need a lookout helper to assist you.

Stacker are for Carrying Loads only

- If a person has to be lifted, use only a securely attached work platform and cage and follow the appropriate operating instructions.

Keep Clear of the Mast

- Do not authorize anyone to stand or walk under the load or Stacker - The load can fall causing injury or death.

- Keep hands and feet clear of the cross members of the mast - Serious injury can be caused if the mast is lowered while your hand is on it.

Stackers are for Carrying Loads only

- It is not allowed to carry people on stacker.

Driving on Ramps

- When driving up ramps’ move in a forward direction and down ramps in reverse, especially while carrying loads.

- Do not load or unload goods or turn whilst on a ramp.

Ensure the powered pallet truck is not Over-loaded

- Know the capacity of your stacker and any attachments being used and never exceed this capacity.

- An overload can cause the rear tyres to be raised off the ground and may cause the stacker to tip over.

Ensure the Load is evenly distributed

- Do not lift or move a load unless both forks are fully under the load.

- Do not lift a load with one fork. Use pallets and skids that can withstand the weight of the load.

- Do not use damaged, deformed or decayed pallets for holding loads.

Refueling and charging

- A powered pallet truck should only be charged at specially designated locations.

- Switch off the powered pallet truck.

- For IC engine stacker, no open flame or sparks are permitted, and refueling/ charging should take place in a well-ventilated area.

When the Shift Ends

- After use ensure the powered pallet trucks is parked in a designated or authorized area.

- Fully lower the forks and park.

- Turn the stacker “off” and remove the key.

- Do not leave the powered pallet trucks whilst unattended.